Permit #:

Date:

Location:

Inspector:

Plumbing Inspection Checklist – Underground

- Township stamped plans are onsite.

- Pipe is installed as per specification.

- Drainage pipe has not been covered and is supported underneath.

- Drainage pipe is sloped.

- Cleanout has been installed where main drain exits the building.

- Cleanouts are installed where pipe changes direction > 45 deg.

- Floor drain(s) have been installed.

- Trap seal primers for floor drain P-Traps are verified.

- Drainage and vent piping are capped and pressurized with 5 PSI for 15 minutes (OR) drains are filled with water with a minimum water column of 10 Ft. (3m)

- Water supply lines are approved for underground use.

- Underground water supply lines are air tested to 100 PSI for 2 hrs min.