Permit #:

Date:

Location:

Inspector:

Pre-Backfill Inspection Checklist

- Township stamped plans are onsite.

- All debris, rubbish, rocks etc. removed from area to be backfilled.

- Temporary bracing, and/or floor deck is installed.

- Cold room/crawlspace ventilation is installed.

- Perimeter insulation is complete (if required).

- Damp-proofing/parging for concrete block is complete.

- The top cores of concrete block are filled with concrete/anchor bolts.

- ‘Dimple board’ or other backfill protection material is installed.

- Membrane/waterproofing is installed on ICF and extends onto the footing.

- Weeping tile is installed, including any window well drains, 4” minimum.

- Weeping tile extends away from the foundation and is supported.

- Minimum of 6” of clear stone is installed over weeping tile.

- Landscape fabric, silt cloth or ‘socked’ weeping tile is complete.

- Any field reviews or soils reports from Engineers are received.