Permit #:

Date:

Location:

Inspector:

Final/Occupancy Checklist

Required Paperwork

- Township stamped plans are onsite.

- Septic Final

- ESA final from the Electrical Safety Authority. (Electrician)

- All structural Engineering reports submitted (if required)

- W.E.T.T. installation reports for woodstoves/fireplaces.

- Welder’s Certificate for any structural steel that was welded.

- Balancing report for HVAC.

- Insulation certificates, fireproofing certificates for spray foam.

- Manufacturer’s specs/Engineering for custom guards.

Exterior

- All siding/cladding complete, up to the roof.

- Roofing and venting is installed.

- Flashing/drip cap installed at all windows and doors.

- Caulking at windows/doors/mechanical penetrations completed. *

- Entrance lighting is installed and operational.

- Entry doors have windows/sidelights/locksets/peepholes.

- Foundation is a minimum of 6” above grade, including deck footings.

- Final grading is completed and sloping away from foundation.*

- Exposed exterior ICF foundation Styrofoam has been parged.

- Decks are complete with required guards and handrails installed.

- Exterior guards are as per SB-7 and have no openings over 4”.

- Hose bibs are installed with backflow preventers at all outdoor taps.

- Doors to attached garages are weather stripped and have self-closers.

- Weep holes are provided in all bottom courses of masonry cladding.

Interior

- Smoke alarms are installed as per plans, interconnected and functional.

- Handrails are installed with a minimum of 2” clearance to the walls.

- Interior and exterior handrails are continuous with no interruptions.

- Guards for stairs are not ‘climbable’ with no openings greater than 4”.

- Stairs are consistent in rise and run throughout.

- Headroom above stairs is a minimum of 76 ¾”.

- Lighting is installed and operational in garages and storage rooms.

- Bathroom doors are installed on all bathrooms.

- Waterproof walls/floors are completed at all showers and tubs.

- Flooring is completed throughout. (houses)

- Attic hatches are installed and weather stripped.

- Clearances above and beside stove (min. 17 ¾” side, 29 ½” above)

- All exposed foam insulation is covered with suitable thermal barrier.

Plumbing and Mechanical

- Heating, plumbing and ventilation systems are complete and operational.

- Furnace, HRV, water heaters, windows and doors meet energy efficiencies.

- Water heaters have a tempering (mixing) valve and safety blowdown tube.

- Water service and all toilets have accessible shut-off valves.

- Hot and cold controls on faucets are installed with hot (left), cold (right)

- All drain cleanouts are installed and accessible.

- Sump/Sewage basins are completed with secured lids and required valves.

- The first 8 ft. of the hot water line after the tank is insulated. (R-3.5) (houses)

- A programmable thermostat is installed for heating system. (houses)