Permit #:

Date:

Location:

Inspector:

Plumbing Inspection Checklist – Above Ground

- Township stamped plans are onsite.

- Cleanouts are installed at the base of stacks/changes in direction.

- Drainage pipe is sloped to a minimum of 1 in 50, max. 1 in 25.

- Drain water heat recovery unit (DWHR) is installed (year-round homes).

- Drain and vent pipes are supported/clamped every 4 ft. min.

- Drain and vent pipes are capped/charged with air to 5 psi for 15 mins. OR capped and filled with water for 15 minutes with a min. 10’ column

- Waterlines are capped and air tested to 100 psi for 2 hrs. min.

- Waterlines have clips/clamps installed max. 1m/ 3’3” apart.

- Vents are a minimum of 3” size where they exit the roof or building.

- Vents are sloped properly and secured to framing at the building exit.

- Incoming water supply line is ¾” minimum in size. (if installed)

- Basement sewage ejector is installed/vented (2” min. vent size)

- Protection plates are installed where pipes are drilled through studs and top/bottom plates where penetration by screws/nails is possible.

Permit #:

Date:

Location:

Inspector:

Plumbing Inspection Checklist – Above Ground

- Township stamped plans are onsite.

- Cleanouts are installed at the base of stacks/changes in direction.

- Drainage pipe is sloped to a minimum of 1 in 50, max. 1 in 25.

- Drain water heat recovery unit (DWHR) is installed (year-round homes).

- Drain and vent pipes are supported/clamped every 4 ft. min.

- Drain and vent pipes are capped/charged with air to 5 psi for 15 mins. OR capped and filled with water for 15 minutes with a min. 10’ column

- Waterlines are capped and air tested to 100 psi for 2 hrs. min.

- Waterlines have clips/clamps installed max. 1m/ 3’3” apart.

- Vents are a minimum of 3” size where they exit the roof or building.

- Vents are sloped properly and secured to framing at the building exit.

- Incoming water supply line is ¾” minimum in size. (if installed)

- Basement sewage ejector is installed/vented (2” min. vent size)

- Protection plates are installed where pipes are drilled through studs and top/bottom plates where penetration by screws/nails is possible.