Permit #:

Date:

Location:

Inspector:

Framing Inspection Checklist

All framing has been completed as per approved plans, or revisions have been submitted and approved and are onsite.

- Township stamped plans are onsite

- Sill plates are installed with gaskets, anchor bolts tightened.

- Beams complete, installed with joints over posts or proper ¼ points.

- Beam plies are fastened together and have a minimum 3” end bearing.

- Wood and steel beams on concrete have moisture protection (wrap)

- Steel beams and columns are primed, welder’s certificate available.

- Floor joists and subflooring are complete and secured.

- Floor, Deck and Ceiling joists are supported by beams, have hangers or a minimum 1 ½” bearing.

- Blocking/bridging is complete on all joists with spans over 6’11” (2.1m)

- Blocking between floor joists for load bearing/non load bearing walls is complete. (24” o/c for bearing, 48” o/c for non-load bearing)

- Wall studs, point loads and exterior sheathing are complete and secured.

- Double top plates for walls are installed and lapped at corners.

- Lintels over window and door openings are complete as per plans.

- All hangers/hardware are installed and fastened with approved fasteners.

- Columns/support posts are centered on footings and fastened top and bottom.

- Grab bar blocking has been installed in at least one bathroom.

- Roof trusses, rafters, bracing and sheathing are complete and secured.

- Ridge beams/girder trusses are fully supported with point load blocking.

- Eave protection and roofing underlayment is installed on roof plywood.

- Drip-cap and all flashing is installed under or taped to air barrier.

- Guards, handrails and stairs are complete and installed as per plans.

- Finished roofing/venting and windows are installed. (if possible)

- Exterior air barrier/sheathing membranes are complete and taped.